|  | Abdou Moumouni University International Master Program in Energy and Green Hydrogen (IMP-EGH) |  |
| --- | --- | --- |

Model Answers

Power-to-X (PtX)

Part 1: Foundations     Part 2: Pathways     Case-study projects

Worked solutions to the three 1-hour subjects (A, B, C)

with justifications, calculations and the Kassø e-Methanol case study

| Course lecturer Dr Bachir Yaou Balarabe Islamic University of Niger | Prepared by KOUAME Koffi Fidèle IMP-EGH, option System Analysis |
| --- | --- |

October 2026

Answer Key at a Glance
Power-to-X (PtX)

Part A. Multiple-choice questions

| Subject | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | D | A | D | B | C | B | D |
| B | B | A | B | D | B | B | A | B |
| C | D | C | B | A | D | B | A | A |

Part B. True or False

| Subject | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | T | T | F | F | F | T | F | T |
| B | T | F | F | T | T | F | T | F |
| C | T | F | T | F | T | T | F | T |

Model Answers | Subject A
Power-to-X (PtX)

# Part A. Multiple-choice questions (8 × 0.5 = 4 pts)

A1. In “Power-to-X”, the letter X designates:

Answer B, the final product (carrier, fuel, chemical or heat).

Justification. “Power” is the renewable electricity input; “X” is the output: hydrogen or a hydrogen-derived carrier (ammonia, methanol, synthetic methane, liquid fuels), a chemical, or heat.

A2. In a PEM electrolyser, the reaction at the anode is:

Answer D, H2O→12O2+2 H++2 e−.

Justification. The PEM electrolyte is acidic and conducts protons. Water is oxidised at the anode (oxygen evolution); the protons cross the membrane and are reduced at the cathode (2 H++2 e−→H2). Option A is the SOEC anode, option C the alkaline cathode, option B a cathode reaction.

A3. The electrolyte of a solid oxide electrolyser (SOEC) is typically:

Answer A, yttria-stabilised zirconia (YSZ).

Justification. YSZ is a ceramic that conducts O²⁻ ions at 600–1000 °C. Nafion is the PEM membrane, 25–30 % KOH the alkaline electrolyte, and molten carbonate the MCFC electrolyte.

A4. Catalyst and pressure of green methanol synthesis:

Answer D, Cu/ZnO-based catalyst, 50–100 bar.

Justification. Methanol synthesis uses Cu/ZnO/Al₂O₃ at about 200–300 °C and 50–100 bar. An iron catalyst at 150–300 bar describes Haber–Bosch; Co or Fe at low pressure describes Fischer–Tropsch.

A5. Reaction that prepares syngas for Fischer–Tropsch from green H₂ and captured CO₂:

Answer B, H2+CO2→CO+H2O, the reverse water-gas shift (RWGS).

Justification. FT synthesis needs CO, not CO₂. The endothermic RWGS (ΔH≈+41 kJ/mol) converts captured CO₂ into CO, which is then mixed with H₂ to reach H₂/CO ≈2.

A6. A fuel cell with a molten lithium/potassium carbonate electrolyte operates at about:

Answer C, 650 °C.

Justification. The MCFC electrolyte must be molten to conduct CO​32− ions, which requires about 600–700 °C.

A7. Overall power-to-methanol/DME efficiency given in the course:

Answer B, 50–55 %.

Justification. Electrolysis (60–70 % LHV) is followed by synthesis and distillation losses. Check: 0.189 t H₂ per tonne of methanol at about 55 kWh/kg gives 10.4 MWh, against 5.5 MWh of methanol (LHV 19.9 GJ/t), that is about 53 %.

A8. For a plant driven mainly by a solar park, like Kassø, the best-suited electrolyser is:

Answer D, PEM, because of its response in seconds and wide partial-load range.

Justification. Solar output varies within minutes and falls to zero at night. PEM follows these variations; AWE has a minimum load of 15–40 % and slower dynamics, and SOEC needs a steady high-grade heat source.

# Part B. True or False (8 × 0.5 = 4 pts)

| No. | Statement | T/F | Correction or justification |
| --- | --- | --- | --- |
| B1 | Electrolytic hydrogen routinely reaches 99.99 % purity. | T | Electrolysis gives very pure H₂ (99.9 to 99.999 % after drying and deoxidation), suitable for fuel cells. |
| B2 | Direct air capture extracts CO₂ from ambient air at about 420 ppm. | T | The very low concentration is why DAC is the most energy-intensive and costly CO₂ source. |
| B3 | Alkaline electrolysers need platinum-group metal catalysts. | F | Correction: AWE uses non-noble nickel-based electrodes; PEM needs platinum (cathode) and iridium (anode). |
| B4 | Methanation CO2+4 H2→CH4+2 H2O is endothermic. | F | Correction: it is strongly exothermic, ΔH298∘=−165 kJ/mol; heat must be removed. |
| B5 | In an alkaline fuel cell (AFC), water is produced at the cathode. | F | Correction: water is produced at the anode: H2+2 OH−→2 H2O+2 e−. |
| B6 | Biogenic CO₂ supply may be more dispersed and seasonal than industrial point sources. | T | Biogas and bioethanol plants are small, scattered and depend on feedstock seasons, unlike cement or steel plants. |
| B7 | The round-trip efficiency of Power-to-Power (30–45 %) is higher than that of batteries. | F | Correction: it is lower; batteries reach about 80–95 %. Power-to-Power is justified for long-duration and seasonal storage. |
| B8 | Without heat recovery, power-to-SNG efficiency is 50–65 %; with heat recovery it can exceed 70–80 %. | T | Recovering the methanation and electrolyser heat (district heating, process heat) raises the useful-energy efficiency. |

# Part C. Complete the missing information (3 pts)

Electrolyser technology comparison matrix (answers in green).

| Factor | Alkaline (ALK) | PEM | AEM | SOEC |
| --- | --- | --- | --- | --- |
| Electrolyte | Aqueous KOH (25–30 %) | Solid polymer | Solid anion membrane | Ceramic YSZ (O²⁻ conductor) |
| Operating temperature | 60–90 °C | 50–80 °C | 40–70 °C | 600–1000 °C |
| Response time | Slow | Fast | Fast | Slow |
| Maturity | Mature (commercial for decades) | Commercialized | Emerging | Pre-commercial |
| Lifetime (h) | 60,000–90,000 | 30,000–90,000 | 10,000–20,000 | < 20,000 (about 10,000–20,000) |

# Part D. Reactions (3 pts)

D1. Complete and name CO2+4 H2→… (0.75 pt)

CO2+4 H2→CH4+2 H2O  ΔH298∘=−165 kJ/mol

Sabatier reaction (catalytic CO₂ methanation), the core of Power-to-Methane. Nickel-based catalyst (ruthenium as an alternative), about 250–400 °C and 1–30 bar. The reaction is exothermic and reduces the number of gas moles (5 to 3), so it is favoured by moderate temperature and higher pressure.

D2. Methanol to dimethyl ether (DME) and type of reaction (0.75 pt)

2 CH3OH→CH3OCH3+H2O  ΔH≈−23.4 kJ/mol

Catalytic dehydration (condensation) of methanol over a solid acid catalyst (γ-Al₂O₃ or zeolite), about 250–400 °C and 10–20 bar. It is mildly exothermic.

D3. Half-reactions and overall reaction of a PEM fuel cell (0.75 pt)

Anode (oxidation): H2→2 H++2 e−Cathode (reduction): 12O2+2 H++2 e−→H2OOverall: H2+12O2→H2O  E∘=1.23 V, ΔG∘=−237 kJ/mol

Protons cross the polymer membrane; water forms at the cathode; operation at about 60–80 °C.

D4. Kassø synthesis reaction and reactant masses per tonne of methanol (0.75 pt)

CO2+3 H2→CH3OH+H2O  ΔH298∘≈−49.5 kJ/mol

Cu/ZnO/Al₂O₃ catalyst, about 200–300 °C and 50–100 bar.

nCH3OH=106 g32.04 g/mol=31,211 molmH2=3×31,211×2.016=188,800 g≈0.189 tmCO2=31,211×44.01=1,373,600 g≈1.374 t

Each tonne of methanol therefore needs about 189 kg of H₂ and 1.37 t of CO₂, and releases 0.56 t of water. In practice the consumption is slightly higher because of purge losses and side products.

# Part E. Case study: Project 4, Kassø e-Methanol (Denmark) (6 pts)

E1. Name, country, developers or owners and main objective (1 pt)

- Name and location: Kassø e-methanol facility (Kassø Power-to-X), Kassø, near Aabenraa, Southern Jutland, Denmark.

- Developer and owners: developed by the Danish renewable-energy company European Energy, owned 51 % by European Energy and 49 % by Mitsui & Co. (Japan). Electrolysers supplied by Siemens Energy; methanol loop designed and built by European Energy.

- Milestones: first green hydrogen in January 2025, first e-methanol in March 2025, RFNBO certification (ISCC EU) in April 2025, official inauguration on 13 May 2025.

- Main objective: produce up to 42,000 t/yr of e-methanol at the world’s first commercial-scale e-methanol plant, to decarbonise shipping (A.P. Moller-Maersk) and chemical and plastic production (LEGO Group, Novo Nordisk; OMV as additional offtaker), and to demonstrate Power-to-X at industrial scale.

E2. Power-to-X pathway as a chain of steps and core reaction (1.5 pts)

Simplified process chain of the Kassø e-methanol plant (dashed arrows: by-products).

- Renewable electricity: 304 MW Kassø solar park (about 50 %) and grid electricity under a balancing agreement (about 50 %).

- Water treatment to deionised quality, then PEM electrolysis (three units, about 52 MW): 2 H2O→2 H2+O2.

- Hydrogen compression and buffer storage, which decouples variable electrolysis from the synthesis loop.

- Biogenic CO₂ captured at biogas plants (the first batch came from Tønder), purified and delivered.

- Catalytic methanol synthesis in a recycle loop, then distillation to remove water.

- Storage and delivery of e-methanol to offtakers.

Core reaction: CO2+3 H2→CH3OH+H2O (ΔH≈−49.5 kJ/mol). Combined with electrolysis, the net Power-to-Methanol balance is CO2+2 H2O→CH3OH+32O2.

E3. Key technical data and choice of electrolyser (1.5 pts)

| Item | Value |
| --- | --- |
| Capacity | 42,000 t/yr e-methanol (about 53 million litres) |
| Electrolysis | PEM, Siemens Energy, three units, about 52 MW in total |
| Hydrogen | about 6,000 t/yr (project page) to 8,000 t/yr (2025 media kit); stoichiometry for 42,000 t requires 7,930 t |
| Power supply | 304 MW solar park (about 50 %) and grid (about 50 %) |
| Carbon source | Biogenic CO₂ from biogas plants, about 57,700 t/yr at nameplate |
| Product quality | Industry-grade e-methanol, certified RFNBO under the EU scheme ISCC EU |

Why PEM. The plant is driven by solar power, which varies within minutes and stops at night. PEM electrolysers respond in seconds, operate over a wide partial-load range, start quickly, deliver high-purity hydrogen at elevated pressure and are compact. Alkaline electrolysers are cheaper but have a minimum load of 15–40 % and slower dynamics; SOEC requires a steady high-temperature heat source that a solar-driven site does not have. The cost of PEM (platinum and iridium catalysts, ultrapure water) is accepted in exchange for flexibility.

E4. Improvement question: which inputs would you change? (2 pts)

The plant has four inputs: electricity, water, CO₂ and heat. In order of impact:

- Electricity. Hybridise the supply with dedicated wind power purchase agreements. Danish solar has a capacity factor of only about 11–12 %, while wind reaches 30–45 %; a hybrid profile raises electrolyser full-load hours, lowers the cost per kilogram of hydrogen and secures hourly renewable matching, which EU RFNBO rules require from 2030.

- Heat. Recover electrolyser waste heat (30–40 % of the input, at 50–80 °C) and the exothermic synthesis heat for distillation and district heating. This raises the useful-energy efficiency well above the 50–55 % of the methanol product alone.

- Water. Recycle the 0.56 t of process water released per tonne of methanol to the electrolysers. At nameplate this is about 23,600 t/yr, roughly one third of the theoretical demand of about 71,000 t/yr.

- CO₂. Contract several biogenic sources and add liquid CO₂ storage, since biogenic supply is dispersed and seasonal; direct air capture can be a long-term complement.

- By-product oxygen. Sell the oxygen (about 8 kg per kg H₂, around 60,000 t/yr) to wastewater, medical or industrial users to add revenue.

End of Subject A

Model Answers | Subject B
Power-to-X (PtX)

# Part A. Multiple-choice questions (8 × 0.5 = 4 pts)

A1. The first documented Power-to-X projects appeared around:

Answer B, 1990.

Justification. Reviews of demonstration projects list the first plants from the late 1980s and early 1990s (for example Higashi-Hiroshima 1988, Fayetteville 1991, PHOEBUS in Jülich 1993). 1839 is Grove’s fuel cell, not a PtX project.

A2. In an AWE, the electrolyte is typically:

Answer A, 25–30 % aqueous KOH.

Justification. Alkaline electrolysers use a concentrated potassium hydroxide solution and nickel electrodes; OH⁻ is the charge carrier.

A3. Theoretical water consumption of electrolysis:

Answer B, 9 kg H₂O per kg H₂.

Justification. From H2O→H2+12O2: 18.015/2.016=8.94 kg of water per kg of hydrogen. Real plants need about 10–15 kg because of water treatment losses.

A4. Typical operating conditions of Haber–Bosch synthesis:

Answer D, 150–300 bar, 400–500 °C, iron-based catalyst.

Justification. High pressure shifts the equilibrium towards ammonia (4 moles of gas give 2); 400–500 °C gives acceptable kinetics on the promoted iron catalyst. Option B describes methanol synthesis.

A5. CO₂ source with a high concentration (10–30 % or more):

Answer B, industrial point sources (cement, steel, refineries).

Justification. The higher the CO₂ concentration, the lower the capture energy and cost. Air contains only about 420 ppm.

A6. To stay competitive without heavy subsidies, renewable electricity should ideally cost less than:

Answer B, 30–40 €/MWh.

Justification. Electricity is the largest cost item of green hydrogen: at about 55 kWh/kg, each 10 €/MWh adds about 0.55 €/kg H₂.

A7. Why is intermediate H₂ storage recommended between electrolysers and synthesis units?

Answer A, to buffer slower synthesis units while fast electrolysers track variable renewable surplus.

Justification. Synthesis loops (methanol, ammonia, methanation) prefer steady operation and ramp slowly, whereas the electrolyser follows the renewable profile; the buffer decouples the two.

A8. Share of global anthropogenic CO₂ from conventional ammonia production:

Answer B, 1.2 %.

Justification. Conventional ammonia relies on steam methane reforming or coal gasification and accounts for roughly 1–2 % of global CO₂ emissions.

# Part B. True or False (8 × 0.5 = 4 pts)

| No. | Statement | T/F | Correction or justification |
| --- | --- | --- | --- |
| B1 | More than 80 % of current final energy still comes from fossil fuels. | T | Fossil fuels still supply about four fifths of global energy, which motivates Power-to-X for hard-to-electrify uses. |
| B2 | Power-to-Heat is a hydrogen-based PtX route treated in detail in the series. | F | Correction: Power-to-Heat converts electricity directly into heat without hydrogen; the series focuses on hydrogen-based routes. |
| B3 | AWE has a faster dynamic response than PEM. | F | Correction: PEM is faster (seconds); AWE is slower and has a minimum load of 15–40 %. |
| B4 | Ammonia combustion or cracking produces no CO₂, only N₂ and water (NOₓ must be controlled). | T | NH₃ contains no carbon: 4 NH3+3 O2→2 N2+6 H2O. |
| B5 | In a PAFC, water is produced at the cathode and the cell tolerates CO₂ in air. | T | The acid electrolyte conducts H⁺ to the cathode, and unlike the alkaline AFC it does not react with CO₂. |
| B6 | Biological methanation needs 200–400 °C and 30 bar. | F | Correction: biological methanation uses archaea at about 30–70 °C and near-ambient to moderate pressure (1–10 bar); 200–400 °C applies to catalytic methanation. |
| B7 | Additionality means the renewable generation used must be new, not diverted from other consumers. | T | This is the EU RFNBO additionality principle (Delegated Regulation 2023/1184). |
| B8 | Electrolyser flexibility always increases utilisation and therefore always lowers costs. | F | Correction: following variable power often lowers full-load hours and raises capital cost per kg; flexibility lowers cost only when the savings on electricity exceed this effect. |

# Part C. Complete the missing information (3 pts)

Fuel-cell types (answers in green).

| Type | Electrolyte | Mobile ion | Temperature | Water produced at |
| --- | --- | --- | --- | --- |
| AFC | Aqueous KOH | OH⁻ | 60–90 °C | anode |
| PAFC | Liquid phosphoric acid (H₃PO₄) | H⁺ | 150–200 °C | cathode |
| PEMFC | Solid polymer membrane | H⁺ | 60–80 °C | cathode |
| MCFC | Molten Li/K carbonate | CO​32− | 650 °C | anode |
| SOFC | Solid ceramic (zirconia) | O²⁻ | 800–1000 °C | anode |

# Part D. Reactions (3 pts)

D1. Overall water electrolysis and gas formed at the cathode (0.75 pt)

2 H2O→2 H2+O2  ΔH∘=+286 kJ/mol H2 HHV

Reversible voltage 1.23 V, thermoneutral voltage 1.48 V. Hydrogen forms at the cathode (reduction) and oxygen at the anode, in a 2:1 volume ratio.

D2. Half-reactions of alkaline water electrolysis (0.75 pt)

Cathode: 2 H2O+2 e−→H2+2 OH−Anode: 2 OH−→12O2+H2O+2 e−Overall: H2O→H2+12O2

OH⁻ ions migrate through the diaphragm from the cathode to the anode, in 25–30 % KOH at 60–90 °C.

D3. Haber–Bosch reaction, conditions and catalyst (0.75 pt)

N2+3 H2⇌2 NH3  ΔH∘=−92 kJ/mol

Promoted iron catalyst (magnetite with K₂O and Al₂O₃), 150–300 bar, 400–500 °C. Conversion per pass is only about 15–25 %, so unreacted gas is recycled after ammonia condensation.

D4. Steam reforming and water-gas shift, and why they are not used at Kassø (0.75 pt)

Steam methane reforming: CH4+H2O→CO+3 H2  ΔH∘=+206 kJ/mol Ni, 850–950 ∘CWater-gas shift: CO+H2O→CO2+H2  ΔH∘=−41 kJ/mol

These reactions make hydrogen and syngas from fossil methane and release about 9–10 kg of CO₂ per kg of H₂. At Kassø, hydrogen comes from water electrolysis powered by renewable electricity, and carbon comes from biogenic CO₂ that is hydrogenated directly (CO2+3 H2→CH3OH+H2O). No fossil feedstock and no reforming step are needed, which is the condition for RFNBO status.

# Part E. Case study: Project 4, Kassø e-Methanol (Denmark) (6 pts)

E1. Renewable energy sources and their proportion (1 pt)

About 50 % of the electricity comes from the adjacent 304 MW Kassø solar park, one of the largest in Northern Europe. The other 50 % comes from the public grid through a balancing agreement (Danske Commodities); in Denmark this supply is dominated by wind power. The grid share compensates for the low capacity factor of Danish solar (about 11–12 %) and keeps the electrolysers running outside sunny hours.

E2. Environmental benefits and EU condition (1 pt)

Benefits. e-Methanol replaces fossil methanol made from natural gas or coal; it reuses biogenic carbon (short carbon cycle), uses surplus renewable power, and burns without SOₓ and with low particulate and NOₓ emissions, which suits shipping (Maersk) and plastics (LEGO, Novo Nordisk).

EU condition. The product must qualify as a renewable fuel of non-biological origin (RFNBO) under the Renewable Energy Directive:

- at least 70 % greenhouse-gas savings against the fossil comparator of 94 g CO₂-eq/MJ, that is at most 28.2 g CO₂-eq/MJ (Delegated Regulation (EU) 2023/1185);

- renewable electricity rules (Delegated Regulation (EU) 2023/1184): additionality, temporal correlation (monthly until 2029, hourly from 2030) and geographical correlation (same bidding zone);

- certification by a recognised scheme. Kassø obtained ISCC EU RFNBO certification in April 2025, the first such certification for e-methanol.

E3. Mass balance (2 pts)

Reaction CO2+3 H2→CH3OH+H2O, with 1 mol CH₃OH (32.04 g) requiring 3 mol H₂ (6.048 g) and 1 mol CO₂ (44.01 g).

(a) mH2=42,000×6.04832.04=7,928 t H2/yr(b) mCH3OH=6,000×32.046.048=31,786 t CH3OH/yr(c) mCO2=42,000×44.0132.04=57,691 t CO2/yr

Comment.

- 6,000 t of H₂ can only give about 31,800 t of methanol, that is 76 % of the nameplate. The 42,000 t/yr is a maximum capacity; reaching it needs about 7,930 t of H₂, which matches the 8,000 t/yr quoted in the 2025 media kit.

- With 52 MW at about 55 kWh/kg, full-load operation would give about 8,300 t H₂/yr. Producing 6,000 t corresponds to about 72 % of the hours, and 7,930 t to about 96 %: solar alone cannot achieve this, hence the grid supply.

- The plant consumes about 1.37 t of biogenic CO₂ per tonne of methanol, and the same carbon is released when the fuel is burnt: the benefit comes from replacing fossil carbon, not from permanent storage.

- Real consumption is slightly higher than stoichiometry because of purge gas losses and by-products.

E4. Improvement question: which inputs would you change? (2 pts)

- Electricity mix. Add wind PPAs and possibly a battery to raise electrolyser full-load hours (solar capacity factor of about 11–12 % against 30–45 % for wind) and to meet the hourly matching rule from 2030. This acts on the largest cost item, electricity.

- Hydrogen buffer. Enlarge hydrogen storage so the synthesis loop runs steadily while the electrolysers follow the renewable profile.

- Heat and water integration. Use electrolyser and synthesis heat for distillation and district heating, and recycle the 0.56 t of process water per tonne of methanol to the electrolysers.

- CO₂ supply. Diversify biogenic sources and add liquid CO₂ storage to cover seasonal variation.

End of Subject B

Model Answers | Subject C
Power-to-X (PtX)

# Part A. Multiple-choice questions (8 × 0.5 = 4 pts)

A1. Fuel cell used on the Apollo spacecraft:

Answer D, alkaline fuel cell (AFC).

Justification. Apollo used pure H₂ and O₂, so the CO₂ sensitivity of the KOH electrolyte was not a problem; the cells also supplied drinking water.

A2. The fuel cell was demonstrated in 1839 by:

Answer C, William Robert Grove.

Justification. Grove built the “gas voltaic battery”. Schönbein described the effect in 1838; Haber and Sabatier are linked to ammonia and methanation.

A3. Seasonal mismatch, which batteries cannot economically solve, is typical of:

Answer B, mid- and high-latitude climates where winter demand peaks when solar is lowest.

Justification. Batteries store hours to days; shifting energy over months requires chemical carriers such as hydrogen or methane.

A4. The Fischer–Tropsch synthesis uses catalysts based on:

Answer A, Co or Fe.

Justification. Cobalt (low-temperature FT, long-chain waxes and diesel) and iron (high-temperature FT, tolerant to CO₂). Cu/ZnO is for methanol, Ni for methanation, Pt for PEM cells.

A5. Dry reforming of methane is:

Answer D, CH4+CO2→2 CO+2 H2.

Justification. “Dry” means CO₂ replaces steam; the reaction is strongly endothermic (ΔH≈+247 kJ/mol) and gives H₂/CO = 1.

A6. In a SOEC, the species that crosses the electrolyte is:

Answer B, O²⁻.

Justification. Oxide ions formed at the cathode (H2O+2 e−→H2+O2−) cross the YSZ ceramic to the anode.

A7. Most HRES studies still treat PtX as:

Answer A, a simple efficiency black-box.

Justification. Many hybrid renewable energy system models use a fixed conversion efficiency and ignore part-load behaviour, start-up times, degradation and coupling between energy carriers.

A8. Correct statement about Power-to-Power:

Answer A, reversible solid oxide cells can do both electrolysis and fuel-cell operation.

Justification. Its round-trip efficiency is only about 30–45 %, it targets long-duration storage, and it does not require pumped hydro.

# Part B. True or False (8 × 0.5 = 4 pts)

| No. | Statement | T/F | Correction or justification |
| --- | --- | --- | --- |
| B1 | Typical wind and solar capacity factors are 20–45 %. | T | Onshore wind about 25–45 %, solar about 15–25 % in sunny regions (lower at high latitude, about 11–12 % in Denmark). |
| B2 | Ammonia contains carbon. | F | Correction: NH₃ contains only nitrogen and hydrogen; it is a carbon-free carrier. |
| B3 | PEM electrolysers need ultra-pure feed water. | T | Deionised water (conductivity below about 1 µS/cm) protects the membrane and catalysts. |
| B4 | SOEC has the fastest start-up of all electrolysers. | F | Correction: SOEC is the slowest (hours of heating to 600–1000 °C); PEM starts fastest. |
| B5 | FT-SAF is chemically similar to conventional jet fuel and can be used in existing aircraft engines with minimal change. | T | FT synthetic paraffinic kerosene is a drop-in fuel, approved for blends up to 50 % (ASTM D7566). |
| B6 | In the AEM electrolyser, hydroxide ions migrate to the anode. | T | OH⁻ formed at the cathode crosses the anion-exchange membrane and is oxidised to O₂ at the anode. |
| B7 | PtL overall efficiencies of 40–55 % are higher than electrolysis efficiencies of 60–70 % (LHV). | F | Correction: they are lower, because each synthesis step adds losses after electrolysis. |
| B8 | Power-to-X can turn curtailed renewable electricity into new revenue streams and controllable demand. | T | Electrolysers absorb surplus power that would otherwise be curtailed and provide flexibility to the grid. |

# Part C. Complete the missing information (3 pts)

Key reactions for syngas production (answers in green).

| Process | Reaction | Typical temperature |
| --- | --- | --- |
| Steam reforming | CH4+H2O→CO+3 H2 | 850–950 °C |
| Dry reforming | CH4+CO2→2 CO+2 H2 | about 800–1000 °C |
| Partial oxidation | CH4+12O2→CO+2 H2 | catalytic 800–900 °C |
| Water-gas shift | CO+H2O→CO2+H2 | 350–450 °C (Fe–Cr) / 200–250 °C (Cu–Zn) |
| Coal / biomass gasification | C+H2O→CO+H2 | about 800–1000 °C (up to 1400 °C in entrained-flow gasifiers) |

# Part D. Reactions (3 pts)

D1. Half-reactions of a SOEC (0.75 pt)

Cathode: H2O+2 e−→H2+O2−Anode: O2−→12O2+2 e−Overall: H2O→H2+12O2

Steam is fed at 600–1000 °C; part of the energy is supplied as heat, which lowers the electricity demand (about 40 kWh/kg H₂).

D2. Reverse water-gas shift and its role in Power-to-Liquid (0.75 pt)

CO2+H2→CO+H2O  ΔH∘=+41 kJ/mol

Endothermic, favoured above about 700 °C. It converts captured CO₂ into CO, so that green H₂ and CO can be mixed into syngas with H₂/CO ≈2 for Fischer–Tropsch synthesis of e-kerosene and e-diesel.

D3. Direct DME synthesis from syngas (0.75 pt)

3 CO+3 H2→CH3OCH3+CO2  ΔH∘≈−246 kJ/mol

One-step process on a bifunctional catalyst (Cu/ZnO for methanol synthesis combined with a solid acid for dehydration), about 240–280 °C and 30–50 bar. The water-gas shift consumes the water produced, which pushes conversion further.

D4. Methanation versus methanol synthesis (0.75 pt)

|  | Methanation | Methanol synthesis |
| --- | --- | --- |
| Reaction | CO2+4 H2→CH4+2 H2O | CO2+3 H2→CH3OH+H2O |
| Enthalpy | −165 kJ/mol | −49.5 kJ/mol |
| Catalyst | Ni (Ru) | Cu/ZnO/Al₂O₃ |
| Conditions | 250–400 °C, 1–30 bar | 200–300 °C, 50–100 bar |
| H₂ per mole CO₂ | 4 mol | 3 mol |
| Product | Gas (SNG), injectable in gas grids | Liquid, easy to store and ship |

Methanol synthesis uses 25 % less hydrogen per mole of CO₂ and gives a liquid product, while methanation is more exothermic and needs efficient heat removal.

# Part E. Case study: Project 4, Kassø e-Methanol (Denmark) (6 pts)

E1. Main technologies (unit operations) in process order (1 pt)

- Renewable power supply: 304 MW solar park and grid connection, transformers and rectifiers (AC to DC).

- Water treatment: filtration and demineralisation to PEM-grade water.

- PEM electrolysis: three Siemens Energy units, about 52 MW in total.

- Gas conditioning: hydrogen drying, compression and buffer storage; oxygen vented or valorised.

- CO₂ supply chain: capture at biogas plants, purification, liquefaction, transport and storage.

- Feed compression and mixing to the stoichiometric ratio.

- Catalytic methanol synthesis loop (Cu/ZnO) with recycle of unconverted gas and purge.

- Flash separation and distillation to remove water and by-products.

- Product storage and dispatch to offtakers.

E2. Three economic or technical challenges (1 pt)

- Cost competitiveness. Renewable e-methanol cost about USD 800–1,600/t against USD 100–250/t of production cost for fossil methanol (IRENA and Methanol Institute, 2021). Electricity and electrolysers dominate this gap, so long-term offtake contracts (Maersk, LEGO, Novo Nordisk) are essential.

- Variable supply against a steady synthesis loop. Solar has a capacity factor of only about 11–12 % in Denmark, while the methanol loop needs stable operation; this requires grid balancing power, hydrogen buffer storage and careful control.

- CO₂ logistics and regulation. Biogenic CO₂ is dispersed and seasonal, and the plant must continuously prove RFNBO compliance (70 % GHG savings, additionality, temporal and geographical correlation, with hourly matching from 2030).

E3. Comparison with conventional methanol production (2 pts)

| Criterion | Conventional methanol | Kassø e-methanol |
| --- | --- | --- |
| Energy source | Natural gas (feed and fuel, about 30 GJ per tonne) or coal | Renewable electricity, about 10.4 MWh per tonne for the hydrogen (0.189 t H₂ at about 55 kWh/kg) |
| Hydrogen route | Steam or autothermal reforming of methane, coal gasification | Water electrolysis (PEM) |
| Carbon source | Fossil carbon in natural gas or coal | Biogenic CO₂ from biogas plants |
| Emissions | Fossil comparator 94 g CO₂-eq/MJ, that is about 1.9 t CO₂-eq per tonne of methanol over the life cycle | At most 28.2 g CO₂-eq/MJ (at least 70 % savings, RFNBO certified) |
| Scale | World-scale plants of about 5,000 t/day (about 1.7 Mt/yr); global output about 100 Mt/yr | 42,000 t/yr (about 115 t/day), roughly 40 times smaller than a world-scale plant |
| Cost | About USD 100–250/t (production cost) | About USD 800–1,600/t (IRENA, 2021) |

The chemistry downstream of the synthesis gas is the same (Cu/ZnO catalyst, 50–100 bar, distillation); the difference lies in where the hydrogen and the carbon come from. e-Methanol is cleaner but several times more expensive, which is why regulation (RFNBO, FuelEU Maritime) and long-term offtake contracts drive its market today.

E4. Improvement question: prioritised list of input changes (2 pts)

- Electricity, priority 1. Combine the solar park with wind PPAs (and a battery if needed) to raise electrolyser full-load hours, reduce the cost per kilogram of H₂ and secure hourly renewable matching from 2030. Electricity is the largest cost item.

- Hydrogen buffer, priority 2. Increase H₂ storage between the electrolysers and the synthesis loop, so that the loop runs at steady load while the electrolysers follow renewable output.

- Heat, priority 3. Recover electrolyser heat (30–40 % of the input) and the synthesis heat (−49.5 kJ/mol) for distillation and district heating, raising the overall useful-energy efficiency.

- Water, priority 4. Recycle the process water (0.56 t per tonne of methanol, about one third of the theoretical electrolysis demand) and recover water from the distillation column.

- CO₂, priority 5. Diversify biogenic sources and add buffer storage of liquid CO₂; consider direct air capture in the long term.

- Oxygen, priority 6. Sell by-product oxygen (about 8 kg per kg H₂) to industrial or medical users.

End of Subject C

# References

Balarabe, B. Y. (2026). Power-to-X (PtX), Part 1: Foundations and Part 2: Pathways. Lecture notes, International Master Program in Energy and Green Hydrogen (IMP-EGH), Abdou Moumouni University, Niamey.

Chehade, Z., Mansilla, C., Lucchese, P., Hilliard, S., & Proost, J. (2019). Review and analysis of demonstration projects on power-to-X pathways in the world. International Journal of Hydrogen Energy, 44(51), 27637–27655.

European Commission (2023a). Commission Delegated Regulation (EU) 2023/1184 establishing a Union methodology setting out detailed rules for the production of renewable liquid and gaseous transport fuels of non-biological origin. Official Journal of the European Union, L 157.

European Commission (2023b). Commission Delegated Regulation (EU) 2023/1185 establishing a minimum threshold for greenhouse gas emissions savings of recycled carbon fuels and a methodology for assessing greenhouse gas emissions savings from RFNBOs. Official Journal of the European Union, L 157.

European Energy (2025a, 8 April). E-methanol from Kassø gets first ever EU certification on green fuels. https://europeanenergy.com/2025/04/08/e-methanol-from-kasso-gets-first-ever-eu-certification-on-green-fuels/

European Energy (2025b, 13 May). Kassø e-methanol facility officially inaugurated. https://europeanenergy.com/2025/05/13/kasso-e-methanol-facility-officially-inaugurated/

European Energy (2025c). Kassø Power-to-X facility, media kit (20 March 2025) and project page. https://europeanenergy.com/kasso/

IEA (2021). Ammonia Technology Roadmap: Towards More Sustainable Nitrogen Fertiliser Production. Paris: International Energy Agency.

IRENA (2020). Green Hydrogen Cost Reduction: Scaling up Electrolysers to Meet the 1.5 ° C Climate Goal. Abu Dhabi: International Renewable Energy Agency.

IRENA & Methanol Institute (2021). Innovation Outlook: Renewable Methanol. Abu Dhabi: International Renewable Energy Agency.

Thema, M., Bauer, F., & Sterner, M. (2019). Power-to-Gas: Electrolysis and methanation status review. Renewable and Sustainable Energy Reviews, 112, 775–787.